RAM AND STORAGE DEVICES – PRACTICAL ASSIGNMENT

Practical 1: Comparison of DDR3, DDR4 and DDR5 RAM

Aim

To compare DDR3, DDR4 and DDR5 RAM based on speed, voltage, capacity and typical use cases.

Comparison Table

| Feature | DDR3 | DDR4 | DDR5 |
| --- | --- | --- | --- |
| Generation | Older generation | Previous/current widely used generation | Newer generation |
| Typical Speed | 800–2133 MT/s | 1600–3200 MT/s | 4800 MT/s and above |
| Typical Voltage | 1.5 V | 1.2 V | 1.1 V |
| Bandwidth | Lower | Higher than DDR3 | Much higher than DDR4 |
| Typical Capacity per Module | Usually lower | Commonly 4–32 GB | Commonly 8 GB and above |
| Power Consumption | Higher than newer RAM | Lower than DDR3 | Lower operating voltage than DDR4 |
| Typical Use | Older desktops and laptops | Many desktops and laptops | Newer PCs and high-performance systems |
| Gaming Performance | Suitable for older systems | Good for many systems | Better bandwidth for newer systems |
| Compatibility | DDR3 motherboard required | DDR4 motherboard required | DDR5 motherboard required |

Explanation

DDR3:
DDR3 is an older type of RAM. It is commonly found in older computers and laptops. It operates at around 1.5 V in standard configurations.

DDR4:
DDR4 provides higher speed and lower voltage than DDR3. It is widely used in many PCs and laptops and offers a good balance between performance and power consumption.

DDR5:
DDR5 is a newer RAM generation designed for higher bandwidth and newer computer platforms. It starts at much higher data-transfer rates than standard DDR4.

Important Note

DDR3, DDR4 and DDR5 are not directly interchangeable. A motherboard designed for DDR4 cannot normally use DDR3 or DDR5 RAM because the physical design and electrical requirements are different.

Result

The comparison shows that DDR5 provides higher data-transfer speeds and bandwidth, while DDR4 offers a widely used balance of performance and compatibility. DDR3 is mainly suitable for older computer systems.

Practical 2: Removing and Reinstalling a RAM Module

Aim

To safely remove a RAM module from a computer, inspect it, reinstall it and verify that the system boots successfully.

Requirements

- Laptop or desktop computer

- Installed RAM module

- Screwdriver if required

- Clean and dry working surface

- Device manual or trusted installation guide

Safety Precautions

- Shut down the computer completely.

- Disconnect the power adapter/AC power.

- For a desktop, switch off the power supply and unplug the power cable.

- Do not work on the computer while it is powered on.

- Touch a grounded metal surface before handling RAM to reduce static electricity.

- Hold RAM by its edges and avoid touching the gold contacts.

- Do not force the RAM into the slot.

Procedure

Step 1: Shut Down the System

Save all work and shut down the laptop or desktop normally.

Step 2: Disconnect Power

Remove the charger or power cable. For a desktop, disconnect the AC power cable.

Step 3: Open the Device

Open the laptop's RAM access panel or desktop cabinet according to the manufacturer's instructions.

Step 4: Locate the RAM Slots

Find the RAM module installed in the motherboard's memory slot.

Step 5: Remove the RAM

Push the retaining clips on both sides of the RAM slot outward. The RAM module should lift slightly.

Carefully hold the module from its edges and remove it.

Step 6: Take a Photograph

Take a clear photograph showing the RAM module and, if possible, the RAM slot.

Photo 1 – RAM Module

[Insert your actual photograph here]

Observation:
The RAM module contains memory chips and a row of gold contacts used to connect it to the motherboard.

Step 7: Reinstall the RAM

Align the notch on the RAM module with the corresponding key in the RAM slot.

Insert the RAM at the correct angle and press it down gently until the retaining clips lock into position.

Step 8: Close the Computer

Reinstall the laptop cover or close the desktop cabinet.

Step 9: Reconnect Power

Connect the charger or desktop power cable.

Step 10: Start the Computer

Power on the system and wait for Windows to start.

Step 11: Verify RAM

Open:

Settings → System → About

or

Task Manager → Performance → Memory

Check that the installed RAM is detected.

Observation

The RAM module was removed carefully and reinstalled in the correct orientation. After reconnecting the power, the computer successfully booted into the operating system.

Result

The RAM module was successfully removed and reinstalled, and the system booted successfully after reassembly.

Practical 3: Comparison of HDD, SSD and NVMe Storage

Aim

To compare HDD, SSD and NVMe storage and identify suitable everyday applications or use cases for each type.

Comparison Table

| Feature | HDD | SSD | NVMe SSD |
| --- | --- | --- | --- |
| Technology | Magnetic spinning disk | Flash memory | Flash memory using NVMe protocol |
| Connection | Usually SATA | Usually SATA | Usually M.2 PCIe |
| Speed | Slowest | Faster than HDD | Generally fastest of the three |
| Moving Parts | Yes | No | No |
| Durability | More sensitive to physical shock | Better | Better |
| Noise | Can make spinning/seek noise | Silent | Silent |
| Best Use | Large, inexpensive storage | General everyday computing | High-performance tasks |

1. HDD – Hard Disk Drive

An HDD stores data on magnetic spinning disks. It has moving mechanical parts.

Advantages

- Large storage capacity

- Usually lower cost per GB

- Useful for storing large amounts of files

Daily Use Case

Movies, photos and document storage

For example, an HDD can be useful for storing a large collection of videos, photos and backup files where maximum speed is not the main requirement.

2. SSD – Solid State Drive

An SSD stores data using flash memory and has no moving mechanical parts.

Advantages

- Faster than HDD

- Silent operation

- Faster Windows boot

- Faster application loading

Daily Use Case

Windows and everyday applications

Apps such as Chrome, Microsoft Office and other commonly used software can open and respond faster when installed on an SSD compared with an HDD.

3. NVMe SSD

NVMe is a storage protocol designed for high-speed SSDs that commonly connect through PCIe.

Advantages

- Very high transfer speeds

- Low latency

- Fast application and game loading

- Suitable for demanding workloads

Daily Use Case

Gaming and large software projects

For example, large games or development projects can benefit from the faster storage performance of an NVMe SSD.

Result

HDD is suitable for inexpensive large-capacity storage, SATA SSD is suitable for general everyday computing, and NVMe SSD is suitable for high-performance tasks such as gaming, development and large file transfers.

Practical 4: Removing and Reinstalling a Storage Drive

Aim

To identify the storage drive connector type, safely remove and reinstall the drive, and verify that the computer recognizes the storage device.

Requirements

- Laptop or desktop computer

- Screwdriver if required

- Storage drive

- Clean working surface

- Device manual

Safety Precautions

- Shut down the computer completely.

- Disconnect the power cable or charger.

- Do not remove a storage drive while the computer is operating.

- Handle the drive carefully.

- Avoid touching electrical contacts.

- Follow the manufacturer's instructions.

- If the drive contains important data, make sure a backup is available before opening the device.

Procedure

Step 1: Shut Down the Computer

Save all files and shut down Windows.

Step 2: Disconnect Power

Remove the charger or power cable.

Step 3: Open the Computer

Open the laptop's service cover or desktop cabinet according to the manufacturer's instructions.

Step 4: Locate the Storage Drive

Identify the installed storage device.

It may be:

- 2.5-inch SATA HDD/SSD

- M.2 SATA SSD

- M.2 NVMe SSD

Step 5: Identify the Connector

A SATA drive normally uses a SATA data/power connection.

An M.2 NVMe SSD is installed directly into an M.2 slot on the motherboard and is usually secured with a small screw.

Step 6: Take a Photograph

Take a clear photograph showing the connector and storage drive.

Photo 2 – Storage Drive Connector

[Insert your actual photograph here]

Connector Type: __________________________

Storage Type: __________________________

Step 7: Remove the Drive

Remove the screw or connector securing the drive.

Carefully disconnect/remove the drive according to the device manual.

Step 8: Reinstall the Drive

Place the storage drive back into its original position.

Reconnect the SATA cable or insert the M.2 drive into the slot at the correct angle and secure it with the screw.

Step 9: Close the Computer

Replace the back cover or desktop cabinet panel.

Step 10: Reconnect Power

Connect the charger or power cable.

Step 11: Start the Computer

Turn on the system and allow Windows to boot.

Step 12: Verify Storage

Open:

Settings → System → Storage

or

File Explorer → This PC

Check whether the storage drive is visible.

For additional verification, open:

Disk Management

by right-clicking the Start button and selecting Disk Management.

Observation

The storage drive was removed and reinstalled carefully. After powering on the computer, the storage device was detected by the operating system.

Result

The storage drive was successfully reinstalled and the computer recognized the storage device after reassembly.

Alternative Procedure if the Device Cannot Be Opened

If the student does not have permission to open the laptop/desktop or the device is difficult to disassemble, the practical should not be performed physically.

Instead:

- Find a trusted YouTube video for the exact laptop/PC model.

- Watch the complete RAM or storage removal/reinstallation procedure.

- Note the type of RAM slot or storage connector shown.

- Add the video title and URL in the assignment.

- Do not claim that you personally removed the component if you only watched a video.

Video Reference:
Title: __________________________________________

URL: ___________________________________________

Overall Conclusion

In this practical, DDR3, DDR4 and DDR5 RAM were compared based on speed, voltage and typical applications. The correct procedure for removing and reinstalling RAM was studied, including important safety precautions.

HDD, SATA SSD and NVMe SSD were also compared based on technology, speed, durability and everyday use. Finally, the procedure for identifying, removing and reinstalling a storage drive was studied, followed by checking whether the computer successfully detected the drive.

Learning Outcomes

After completing this practical, the student can:

- Identify DDR3, DDR4 and DDR5 RAM.

- Compare RAM speed and voltage.

- Understand basic RAM installation and removal.

- Identify HDD, SATA SSD and NVMe SSD.

- Understand SATA and M.2 storage connections.

- Compare storage performance.

- Safely handle computer components.

- Verify RAM and storage detection in Windows.

- Understand the importance of disconnecting power before hardware work.